# Basic Kinetic Model 1

## Formation of Radicals:

PVC absorbs UV radiation, leading to the formation of radicals.

PVC → R• + Other products

Let [R•] represent the concentration of radicals. The rate of formation of radicals can be given by:
 dR•dt=kfPVC

Here, kf is the rate constant for radical formation, and PVC is the concentration of PVC.

But, the rate of radical formation depends on the UV intensity, I(t) also.

Here kf=βIt

## Quenching of Radicals:

Radicals are quenched by Tinuvin 770, reducing their concentration.

R• + S → RS (quenched product)

The rate of quenching (neutralization) of radicals can be given by:
 dR•dt=-kqR•S

Here, kq is the rate constant for quenching, and S is the concentration of the stabilizer.

## Degradation of PVC:

PVC degrades in the absence of stabilizer.

PVC → Degraded products

dPVCdt=-kdPVC

Here, kd is degradation rate constant of PVC in the absence of stabilizer. Now it modified to include the effect of Tinuvin 770:

kd'=kd1-αS

where α represents the effectiveness factor of Tinuvin 770.

## Radical Recombination:

Radical recombination is another pathway for reducing the concentration of radicals.

dR•dt=-krR•2

where kr is the rate constant for radical recombination.

## Overall Rate Equation:

Combining the two, the overall rate of change of radical concentration is:

dR•dt=βItPVC-kqR•S-krR•2

## Stabilizer Concentration:

Assuming the concentration of the stabilizer remains constant:

dSdt=0

## UV Intensity:

The UV intensity I(t) can vary over time, typically following a periodic function for daily cycles:

It=I0sin2πt/T

where I0​ is the peak UV intensity and T is the period (e.g., 24 hours for a daily cycle).

## Solving differential equations:

## Radical Concentration:

To solve these equations, we can proceed as follows:

dR•dt=βItPVC-kqR•S-krR•2

This is a nonlinear differential equation and typically requires numerical methods for a solution.

## PVC Degradation:

dPVCdt=-kd'PVC

dPVCdt=-kd1-αSPVC

This can be solved analytically:

PVCt=PVC0e-kd1-αSt

## Analytical Approach for Radical Concentration

Due to the nonlinearity introduced by the (krR•2) term, the equation for (R•) is complex. We can simplify it by assuming steady-state conditions or using numerical methods. For steady-state conditions (dR•dt=0), we get:

0=βItPVC-kqR•S-krR•2

Solving this quadratic equation for (R•):

krR•2+kqR•S-βItPVC=0

Using the quadratic formula:

R•=-kqS±kqS2+4krβItPVC2kr

We take the positive root as it represents a physical concentration:

R•=-kqS+kqS2+4krβItPVC2kr

## Summary

### Radical Concentration:

R•=-kqS+kqS2+4krβItPVC2kr

### PVC Concentration:

PVCt=PVC0e-kd1-αSt

This detailed model accounts for the formation and quenching of radicals, radical recombination, and the periodic nature of UV intensity. Solving this model provides insights into the degradation kinetics of PVC and the effectiveness of Tinuvin 770 as a stabilizer under varying UV exposure conditions.

Summary

We use below parameters,

- [PVC]: Concentration of PVC

- [R•]: Concentration of radicals

- [S]: Concentration of Tinuvin 770 (stabilizer)

- kf​: Rate constant for radical formation

- kq: Rate constant for radical quenching by the stabilizer

- kd​: Degradation rate constant of PVC in the absence of stabilizer

- α: Effectiveness factor of Tinuvin 770

- β: Constant representing the rate of radical formation per unit UV intensity

- I(t): Intensity of UV radiation as a function of time

R⋅t=kfPVC/kqS1-e-kqSt --- model 1

R•=-kqS+kqS2+4krβItPVC2kr --- model 2

PVCt=PVC0e-kd1-αSt --- model 1

PVCt=PVC0e-kd1-αSt  ---- model 2