PVC degradation process

Initial Degradation of PVC under UV Light:

~CH₂CHCl~ + hv →  (~CH₂CHCl~)* → ~C H₂CH•~ + Cl•  ---k6

Reaction with Oxygen:

~C H₂CH•~ + O₂ → ~CH₂CHOO•~ ---k7

Formation of Hydroperoxide:

~CH₂CHOO•~ + ~CH₂CHCl~ → ~CH₂CHOOH~ + ~CH=CH~ + Cl•---k8

Further Degradation:

~CH₂CHCl~ + Cl• → HCl + ~CH=CH~ + Cl•--- k9

Decomposition under UV Light:

~CH₂CHOOH~ + hv → ~CH₂CHO•~ + •OH

Reaction with Hydroxyl Radical:

~CH₂CHO•~ + •OH → ~CH₂C=O~

Action of HALS in PVC Degradation

Initial Interaction with Peroxy Radicals

R−N−H + R’OO• → R−N−OH ---k1

where R represents the hindered amine structure.

Conversion to Nitroxyl Radical

R−N−OH + R’OO• → R−NO• + R’OOH ---k2

Reaction with Alkyl Radicals

R−NO• + R’• → R−N(O−R’) ---k3

Reaction with Peroxy Radicals

R−N(O−R’) + R’OO• → ROOR’ + R−NO• ---k4

Regeneration of N-Hydroxy HALS

R−N(O−R’) → R−N−OH + olefin ---k5

Differential equations

If we consider I (the UV intensity) to be constant, we can simplify the differential equation for PVC concentration. So we can write,

K6=βI

Without stabilizer,

Concentration of PVC:

d[PVC]/dt = -k6 [PVC] - k8 [PVC] [R’OO•]-k9[Cl•][PVC]

Concentration of Alkyl Radicals (R’•):

d[R’•]/dt=k6 [PVC]-k7[R’•]

Concentration of Peroxy Radicals (R’OO•):

d[R’OO•]/dt=k7[R’•]-k8[R’OO•][PVC]

Concentration of Peroxy Radicals (Cl•):

d[Cl•]/dt=k6[PVC]+k8[PVC][R’OO•]

With stabilizer,

Concentration of PVC:

d[PVC]/dt = -k6 [PVC] - k8 [PVC] [R’OO•]-k9[Cl•][PVC]

Concentration of Peroxy Radicals (Cl•):

d[Cl•]/dt=k6[PVC]+k8[PVC][R’OO•]

Concentration of N-Hydroxy HALS (R−N−OH):

d[R−N−OH]/dt=k1 [R’OO•][R−N−H]+k5 [R−N(O−R’)]-k2 [R−N−OH][ R’OO•]

Concentration of Alkoxyamine ( R−N(O−R’) )

d[R−N(O−R’)]/dt=k3 [R−NO•][ R’•]-k5[R−N(O−R’)]-k4 [R−N(O−R’)][ R’OO•]

Concentration of Nitroxyl Radical (R−NO•):

d[R−NO•]/dt=k4[R−N(O−R’)][ R’OO•]+k2[R’OO•][ R−N−OH]-k3[R’•][ R−NO•]

Concentration of Alkyl Radicals (R’•):

d[R’•]/dt=k6 [PVC]-k7[R’•]-k3[R’•][ R−NO•]

Concentration of Peroxy Radicals (R’OO•):

d[R’OO•]/dt=k7[R’•]-k8[R’OO•][PVC]-k1[R’OO•][ R−N−H]-k2[R’OO•][ R−N−OH]-k4[R−N(O−R’)][ R’OO•]

k1= 51M-1s-1

k2=5.5X102 M-1s-1

k3= 1.2 X109 M-1s-1

k4=1.9 X10-3 M-1s-1

k5=9.3 X10-5 s-1

k6= βI= 1 x 10⁻⁵ s⁻¹ (assuming moderate UV intensity and a typical PVC formulation)

k7=1 x 10⁸ M⁻¹s⁻¹

k8=0.5 M⁻¹s⁻¹

k9=0.1 M⁻¹s⁻¹